Operation Philosophy of Belt Changing Device

Belt changing devices are specialized equipment designed to efficiently and safely replace conveyor belts, which are essential in many industrial operations. Here's a breakdown of how they generally work:

Core Function:

- The primary purpose is to replace worn-out or damaged conveyor belts with new ones, minimizing downtime and maximizing efficiency.

Key Components and Operation:

- Winder/Coiler:

  - This component is responsible for winding up the old conveyor belt.

  - It typically uses an electric motor and gearbox to provide controlled and consistent winding.

  - The winder often includes a drum or reel to collect the old belt.

- Un-winder/De-Coiler:

  - This component holds the new conveyor belt and dispenses it as the old belt is being wound.

  - It may have a drive system to synchronize the speed with the winder.

  - It also utilizes a drum or reel to hold the new belt.

- Drive System:

  - Electric motors and gearboxes provide the power to wind and unwind the belts.

  - Variable frequency drives (VFDs) are often used to control the speed and ensure smooth operation.

  - Synchronisation of the winder and un winder is very important.

- Control Panel:

  - This allows operators to control the device, including speed, direction, and emergency stops.

Operational Process:

- Preparation:

  - The old conveyor belt is cut at a suitably identified location at one end.

  - The old belt is attached to the winder, and the new belt drum, generally loaded with new belt, is attached to the un-winder.

- Winding and Unwinding:

  - The winder begins to wind the old belt, while the un-winder simultaneously dispenses the new belt.

  - The synchronized drive system ensures that the new belt is laid in place as the old belt is removed.

- Splicing:

  - Once the new belt is in place, the ends are spliced together to create a continuous loop.

- Completion:

  - The equipment is removed, and the conveyor system is ready for operation.

There are two selection mode one is Auto mode another is Manual mode. Here we will discuss about only Auto mode.

After power Up the control panel, once we select the Auto Mode-through Selector switch. Motor shall run a with a default RPM (set by the USER-input/OEM).

Case-1: When the winder will be winding the belt, winder dia will increase gradually. Initially winder will rotate as per default RPM. As soon as the dia increase motor RPM will be reduce through VFD to maintain a Belt winding speed 5-10 mtr/min.

To keep constant speed one encoder is installed at the Guide Roller to get the actual RPM feedback.

Once the RPM will be more than set RPM, VFD will reduce the Motor RPM & keep the winding speed same. This means the RPM of Guide roller shall remain same.

Case-2: Sometimes encoder feedback interrupted due to belt sagging. That time a 30sec waiting is required, after wards if encoder still fails to provide feedback, PLC will give instruction to VFD to run default RPM again until it gets the actual feedback from the Encoder. Once encoder feedback is restored, the VFD shall adjust the RPM to maintain the Belt speed at 5-10 mtr/Min.

Safety Features:

Emergency Stop Switches at coiler.

Manual Override option for precise control during critical operations.

Optionally parking brake to prevent unwanted drum rotation.